# Data Communication And Networking

Date: 19.02.21

Name: RAJUL DUBEY

SAP ID: 500069424

ROLLNo.: R171218081

Task: 1 with the help of server.

Device Requirements:

Router 1,Server 1,swtiches 2, pcs n

Step 1:   Connect both the switches to the router

Step 2: Connect a Server to switch 1

Step 3: Connect pcs to switches 1 and 2

Step 4: Now configure the Router to stablish a connection b/w Switch 1 and Switch 2

Through Router’s CLI

Commands:

en

conf t

int f0/0                   //for Ethernet cable 0 connected switch

ip add 192.168.1.1 255.255.255.0  (for exp)

no shut

exit

int f0/1                //For Ethernet cable 1 connected switch

ip add 192.168.2.1 255.255.255.0

no shut

exit

Step 5: Now configure the Server

Click on the Server

i.)

config->Global->setting

default gateway: 192.168.1.1

ii.)

config->FastEthernet0

Ip Configuration:

Static

Ipv4:	192.168.1.2

255.255.255.0

iii.)

Services->DHCP->

Step 6: Now Configure the router to distribute ips in both switches

*Router’s CLI

Commans:

en

conf t

int f0/0

ip helper-address 192.168.1.2 (Server’s ip)

no shut

exit

int f0/1  (For another router)

ip helpter-address 192.168.1.2

no shut

exit

Step 7:

Now go to any pc of any network and go to IP Configuration

And click on DHCP

Now your pc will get a ip Dynamically

Task: 2 Use Router as a Server.

Step 1: Make the arrangement like picture

above

Step 2: Configure the router through CLI

Commands:

en

conf t

int f0/0    (Through which it’s connected)

ip add 192.168.1.1 255.255.255.0

no shut

exit

ip dhcp pool dotOne (pool name)

default-router 192.168.1.1   (DG)

network 192.168.1.0 255.255.255.0

exit

int f0/1    (Through which it’s connected)

ip add 192.168.2.1 255.255.255.0

no shut

exit

ip dhcp pool dotTwo(pool name)

default-router 192.168.2.1   (DG)

network 192.168.2.0 255.255.255.0

exit

do wr

Step 3: Now go to any pc of any network and go to IP Configuration

And click on DHCP

Now your pc will get a ip Dynamically